Validation Laboratory Testing

Week of February 19, 2018

# Configuration/Integration Of Units

Codha RSU/OBU

- Reached out to Codha support for user manuals

- Downloaded all manuals including the newest firmware

- Both Units are shipped without a Static IP and a DHCP Server is needed

- Configured IP for RSU: 192.168.0.30

- Configured IP for OBU:192.168.0.40

- When configuring IP’s for both units an SSH Session is required

- Ensured all antennas on both the RSU and OBU were plugged in

- Cohda recommended we upgrade the firmware in the RSU and OBU, using an SSH Session, to mk5-15.Release.69988-typical-RSU.

- Using WinSCP we dropped the img file in the appropriate folder

- Ran the following command in a SSH Session:

# fim -u /home/user/mk5-15.Release.69988.img

- Verified the image was installed and marked with a “P” (pending):

# fim -l

- Ran the “reboot” command to reboot unit:

# reboot

- Verified the upgrade copy process was successful:

# fim -l

- Verified both units were receiving GPS Signal with the following command:

# gpspipe -r

- Downloaded the latest version of the application for the RSU and OBU and using WINSCP dropped it into the appropriate folder:

- Once the package was transferred, opened an SSH Session and run the following commands to start the RSU Application:

- The following shows the SPaT and MAP messages being Transmitted and received by the RSU:

- The following command should be used to start the application on the OBU:

- A log directory will be created in “/mnt/ubi/log/current” that contains the transmit and receive packets as well as the GPS information and this can be viewed from the WINSCP Application

- Open the latest log and right click on tx_r1c.pcap and select open, wireshark will display the BSM packets that were broadcasted:

- Open the latest log and right click on rx_r1a.pcap and select open, wireshark will display the TIM packets that were received from the RSU:

- Open the latest log and right click on rx_r1c.pcap and select open, wireshark will display the SPaT packets that were received from the RSU: