Validation Laboratory Testing

Week of May 28, 2018

# Testing Of Units

Cohda RSU - Remote session with Vendor, Daniel Kiel, 05/30/18

- Powered up Cohda RSU

- Checked for GPS

  - Command: gpspipe -r

- Remote session with Vendor, Daniel Kiel, 05/30/18 (see recording of session)

  - Objective was to verify that the Controller can send SPaT (not using Example1609)

    - Went into persistent memory and removed example1609

    - Command: “chronyc tracking” to check the reference time UTC

- Connected to Naztec controller to verify SPaT

  - Copied files locally

  - Stopped the application to change the configuration

  - Deleted “user.conf” which forces a refresh

  - Applied manual changes; disabled security

  - Restarted device with the initial set-up done

  - Enabled logging

  - With no controllers plugged in, SPaT was still being broadcasted.

  - Verified example1609 was not running

  - Looked for configuration parameters that were not default

- Plugged back into three (3) controllers – Naztec, Siemens M60 and Intelight X3 however had no success getting SPaT from either controller. The data is empty.

- Emailed .pcap files to Daniel at his request for his further investigation.